Three Phase LV CT Postpaid Metering Unit

MU33 Metering Unit

Brief Introduction

MU33 Series metering unit is specially designed for the large power user and can achieve the postpaid function. It is equipped with high precision smart energy meter, which has a plug and play communication module for remote reading and controlling; current transformers and Box for outdoor usage. It is a ideal product for Utilities and Distribution company to reduce energy loss and revenue protection. It is suitable to be used for villa, commercial and industrial projects.

Connection Diagram

Feature

Separate Chamber for meter and CT

Plug and play communication module

High accuracy class

Maximum demand

Outdoor usage

Sealable and lockable

Anti-tamper

OUTLINE DRAWING

| SPECIFICATIONS ON METER |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| DESCRIPTION |  | PARAMETER |  |  |  |
| Accuracy |  | Active class 0.5s, reactive class 2.0 |  |  |  |
| Rated Voltage |  | 3x230/400V.3X240/415V |  |  |  |
| Operation voltage range |  | 0.7Un-1.2ur |  |  |  |
| Current |  | 200-800/5A |  |  |  |
| Power consumption |  | Voltage circuit (Un): 2W/5VA Current circuit (in): 0.5VA |  |  |  |
| Protection class |  | IP54 |  |  |  |
| RTC |  | <is/day (in 23℃) |  |  |  |
| SPECIFICATIONS ON METERING UNIT |  |  |  |  |  |  |
| 200A metering unit |  | 500A metering unit |  |  |  |
| CT ratio | 200/5 | CT ratio | 500/5 |  |  |
| Box dimension | 850mmx240mmx160mm | Box dimension | 850mmx240mmx180mm |  |  |
| Weight | Approx. 14kg | Weight | Approx. 18kg |  |  |
| Box material | Stainless steel 201, cold steel | Box material | Stainless steel 201, cold steel |  |  |
| IP protection class | 54 | IP protection class | 54 |  |  |
| 300A metering unit |  | 800A metering unit |  |  |  |
| CT ratio | 300/5 | CT ratio | 800/5 |  |  |
| Box dimension | 850mmx240mmx160mm | Box dimension | 850mmx240mmx180mm |  |  |
| Weight | Approx. 15kg | Weight | Approx. 19kg |  |  |
| Box material | Stainless steel 201, cold steel | Box material | Stainless steel 201, cold steel |  |  |
| IP protection class | 54 | IP protection class | 54 |  |  |